## Housing & Community Development’s

## Housing Inspection Office

36 N. Center Street

Marshalltown IA  50158-5001

Housing Inspections – Tel – (641) 754-6582

Fax – (641) 754-5742

Becky Deemer, Ext. 3105 or (641) 750-3081

bdeemer@marshalltown-ia.gov

Recent Consistent Rental Housing Code Fail Items – 2023

The following is a list including some of the most common fail items found during housing inspections.  This list is not all encompassing, but does include the most frequent fail items and can be used as a reference.  For specific questions, please contact the Housing Inspectors at 641-754-6582.

Smoke Detectors & Fire Extinguishers & Carbon Monoxide Detectors:

- Smoke detector removed or batteries unhooked/removed.  If detector is chirping, replace battery.

- Hard wired detectors missing/unhooked battery, hanging from wiring, detector unhooked from wiring or one is not sounding alarm when another detector is sprayed (inter-connected).

- All smoke detectors must have at a minimum a UL listing 217 (Underwriters Laboratory) on it. They are required in each bedroom, along with one alarm on every level.

- Fire extinguishers not serviced or replaced on a yearly basis as required.  One in kitchen area on path of exit and one in basement near staircase.  All must be installed so visible and accessible.  Not inside cabinets, closets or behind closed doors.

- Fire extinguisher is a BC instead of the required 1-ABC.

- If needed, UL listed carbon monoxide detectors installed on each level according to package directions.

Electrical now required for all rentals

- All outlets within 6 feet of a sink, tub, shower, toilet’s edge and washing machine must be a GFCI outlet or on a GFCI protected circuit with each protected outlet labeled GFCI protected.

- All living rooms & family rooms are recommended or might be required to have at least 2 properly testing 3-prong outlets or 3-prong GFCIs.  This is due to many of today’s electronics coming with grounded plugs and fire & electrical codes prohibit the use of adapters.  This applies to older homes with older wiring.

- All bedrooms and other rooms such as dining rooms are recommended or might be required to have at least 1 properly testing 3-prong outlet or 3-prong GFCI.  This is due to many of today’s electronics coming with grounded plugs and fire & electrical codes prohibit the use of adapters.  This applies to older homes with older wiring.

Typical Electrical Code Violations:

- GFCI outlets (ground fault circuit interrupter) that no longer trip off or trip buttons that are painted over.

- 3-prong appliances, power strips or 6-plexes with ground plug removed or plugged into adapter and then plugged into 2-prong outlets.

- Fixed appliances such as microwaves, refrigerators, stoves, window air conditioners and garage door openers that plug into a power strip.  Must plug directly into properly testing 3-prong outlets or 3-prong GFCIs.

- Light fixtures missing light bulbs or with burned out bulbs or even missing their covers resulting in a fail or inconclusive.

- Frayed electrical cords, taped or spliced cords to electrical items such as lamps, etc.

General Health & Safety Issues:

- Bedroom windows blocked for rescue/egress.

- Cannot store combustibles within 3 feet of furnace, water heater, clothes dryer, electrical panel or water meter.

- Infestation (Cockroaches, fleas, mice, bed bugs, etc.).

- Site conditions such as large amount of garbage and/or debris either inside or outside of rental property.  Presence of unlicensed or junk vehicles.  Or units that have large amount of trash and debris in them.  This would also include poor housekeeping.

- Garbage cans without tight fitting lids and no regular garbage service in place.

- Doors that are 20-minute fire rated door that self closes with a UL listed closer but no longer securely latches such as buildings with multiple units and have interior entrance doors and/or doors to basements or units with doors to attached garages.  Or the doors are missing their labels which means their rating is now expired.

- Deteriorating paint when there are children under the age of six living at unit or visiting there regularly.

Miscellaneous:

- Doors with loose or broken off doorknobs/locks and kitchen or bathroom cabinets with broken or missing drawers or doors.